JS Air Curtains Optima Recessed

Air Curtain Specification Summary

- The Air Curtain shall have a self-supporting casing made of galvanized plated steel. The grille will be finished in structural epoxy-polyester RAL 9016 as standard. Other RAL colours shall be available on request.

- The Air Curtain shall use low noise twisted cross-flow fans driven by an external rotor motor.

- The fans shall be capable of 2 speed selections.

- The Air Curtain shall be available in a choice of sizes nominally 1.0m, 1.5m & 2.0m.

- The Air Curtain shall measure 259mm high and 446mm deep.

- The unit shall be suitable for ceiling recessed mounting using M8 threaded drop rods.

- The inlet grille and blow out jet shall be integrated into a single aluminium frame. For servicing, in normal environments, the grille only has to be periodically wiped or vacuumed.

- The Air Curtain shall be available with a choice of water, electric heating or ambient unheated.

- Water heated units will be suitable for a nominal 80C flow. An optional coil shall be available on request suitable for 60C flow.

- Optional control valves shall be available for water heated units. Valves shall either be on / off control or 0-10V modulating control where required. All valves shall be controlled directly by the Air Curtain.

- Electrically heated units will use 400V 3 phase, or 230V single phase shielded elements with 3 heat settings with power contactors included.

- The Air Curtain will use anodised aluminium blow-out vanes, airfoil shaped.

- The Air Curtain shall include as standard the Advanced Pro controller incorporating an LCD screen, 7m of telephonic cable with RJ11 (Plug & Play) connectors and a hand held infra-red remote.

- The Advanced Pro controller shall allow for automatic control with manual override.

- The Advanced Pro controller shall incorporate a built in temperature sensor. An optional temperature sensor shall be available if the controller is to be located away from the doorway.

- One door contact shall be included. Additional door contacts shall be available on request. An adjustable door delay from 0 to 90 seconds shall be available.

- Maximum speed and heat settings for door open and door closed states shall be available.

- A Boost mode shall be available to increase temperature set point when the door is open.

- A maintenance due indicator shall be shown on the controller when servicing is due.

- The Advanced Pro controller shall be capable of BMS/PLC/Timer etc. enable and disable by a suitable volt free contact.

- Heat enable and disable shall also be available by using a suitable Volt free contact.

- The system is to be supplied by:

JS Air Curtains

City Gates

2-4 Southgate

Chichester

West Sussex

PO19 8DJ

Tel: +44 (0) 1903 858 656

e-mail: sales@jsaircurtains.com		website: www.jsaircurtains.com